# Laboratory Report #3

Name: 	Josh Ratificar						Date Completed:  09-23-2023

Laboratory Exercise Title: Structural Modeling of Combinational Circuits

Block Diagrams:

Above includes the block diagrams for this Laboratory Exercise. Based on the diagrams, the following can be concluded:

2x4 Decoder

3-Input Ports:

- A[0], A[1], E

4-Output Ports:

- D[0], D[1], D[2], D[3]

3x8 Decoder

4-Input Ports:

- A[0], A[1], A[2], E

8-Output Ports:

- D[0], D[1], D[2], D[3], D[4], D[5], D[6], D[7]

Exercise 3A:

Table 1.0 – Truth Table for a 2x4 Decoder

| INPUTS |  |  | OUTPUTS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E | A0 | A1 | D0 | D1 | D2 | D3 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 |

Boolean Expressions:

D3 = A0’A1’E

D2 = A0’A1E

D1 = A0A1’E

D0 = A0A1E

Figure 1.0 – Decoder2x4.v Script

Figure 1.1 – Decoder2x4.v Analysis and Elaboration Test Results

Figure 1.2 – 2x4-Decoder RTL View Output

Figure 1.3 – tb_Decoder2x4.v Script for Testing Module

Figure 1.4 – 2x4-Decoder RTL Simulation Output

Figure 1.5 – 2x4-Decoder Test Bench Monitor Output (Annotations to Figure 1.4)

Discussion of Results (Exercise 3A)

From what we can observe, the circuit is operating how it is expected. This is apparent by observing Figure 1.5’s results in comparison to Table 1.0’s truth table. The “$monitor” command reflects the observable values in Figure 1.4 and thus confirms that the test bench is appropriately designed. The output of this circuitry is dependent on the enabler. When the enabler is off, the output is 0.

Exercise 3B:

Table 2.0 – Truth Table for a 3x8 Decoder

| INPUTS |  |  |  | OUTPUTS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E | A0 | A1 | A2 | D0 | D1 | D2 | D3 | D4 | D5 | D6 | D7 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Figure 2.0 – Decoder3x8.v Script

Figure 2.1 – Decoder3x8.v Analysis and Elaboration Test Results

Figure 2.2 – 3x8-Decoder RTL View Output

Figure 2.3 – tb_Decoder3x8.v Script for Testing Module

Figure 2.4 – 3x8-Decoder RTL Simulation Output

Figure 2.5 – 3x8-Decoder Test Bench Monitor Output (Annotations to Figure 2.4)

Discussion of Results (Exercise 3B)

By modularizing, a 3x8 was created with the use of two 2x4-decoders. By creating a test bench (Figure 2.3), it was straight-forward verifying the results of the RTL simulation (Figure 2.4). As shown by the “$monitor” command, the results of Figure 2.5 reflect the behaviour observed in the truth table (Figure 1.0). As observed, the output is dependent on the enabler being on, which is the desired outcome of the enabler.