# Laboratory Report #6

Name: 	Josh Ratificar						Date Completed:  12-08-2023

Laboratory Exercise Title: Behavioral Modeling of Sequential Circuits

Block Diagrams:

Figure 1.0 – Hex_to_7_Seg_Decoder Block Diagram

Figure 2.0 –Counter_4bit Block Diagram

Exercise 5A:

Figure 1.1 –JK_FF.v Script

/************************************

*   FILE:               JK_FF.v

*   AUTHOR:             Mohan Francis, Josh Ratificar

*   Class:              Gr.3 CpE 3101L Introduction to HDL

*   Group/Schedule      Friday, 7:30 AM to 10:30 AM

*   Description:        JK_FF.v file

************************************/

module JK_FF (

input wire J,K,Reset, Clk,

output reg Q, Q_bar

);

always @(negedge Clk, posedge Reset)

begin

if(Reset)

begin

Q <= 1'b0;

Q_bar <= 1'b1;

end

case({J,K})

2'b00:                          //NO CHANGE

begin

Q <= Q;

Q_bar <= Q_bar;

end

2'b01:                          //RESET

begin

Q <= 1'b0;

Q_bar <= 1'b1;

end

2'b10:                     //SET

begin

Q <= 1'b1;

Q_bar <= 1'b0;

end

2'b11:                          //TOGGLE

begin

Q <= ~Q;

Q_bar <= ~Q_bar;

end

endcase

end

endmodule

Figure 1.2 – JK_FF.v Analysis and Elaboration Test Results

Figure 1.3 – JK_FF RTL View Output

Figure 1.4 – tb_ JK_FF.v Script for Testing Module

/************************************

*   FILE:                   tb_JK_FF.v

*   AUTHOR:             Mohan Francis, Josh Ratificar

*   Class:              Gr.3 CpE 3101L Introduction to HDL

*   Group/Schedule      Friday, 7:30 AM to 10:30 AM

*   Description:        Test Bench for JK_FF.v file

************************************/

`timescale 1 ns / 1 ps

module tb_JK_FF();

reg  J,K,Reset,Clk;

wire Q,Q_bar;

JK_FF UUT(

.J(J),

.K(K),

.Reset(Reset),

.Clk(Clk),

.Q(Q),

.Q_bar(Q_bar)

);

initial

Clk = 1'b0;

always

#5 Clk = ~Clk;

initial begin

Reset = 1'b1;       #10

Reset = 1'b0;

end

initial begin

$display("Test Bench | JK FLIP FLOP...");

J = 1'b0;   K = 1'b0; #5

J = 1'b0;   K = 1'b1; #5

J = 1'b1;   K = 1'b0; #5

J = 1'b1;   K = 1'b1; #5

J = 1'b0;   K = 1'b0; #5

J = 1'b0;   K = 1'b1; #5

J = 1'b1;   K = 1'b0; #5

J = 1'b1;   K = 1'b1; #5

J = 1'b1;   K = 1'b1; #5

$stop;

end

initial begin

$monitor("Time = %2d ns | J = %b | K = %b | CLK = %b | RESET = %b | Q = %b | Q_bar = %b", $time, J, K, Clk, Reset, Q, Q_bar);

end

endmodule

Figure 1.5 – JK_FF RTL Simulation Output

Figure 1.6 – JK_FF Test Bench Monitor Output (Annotations to Figure 1.5)

Discussion of Results (Exercise 5A)

It appears that the simulation is working as expected according to the JK Flip Flop’s behaviour as seen in Figure 1.5. The behaviour expected is that we can set, reset, no-change, and toggle. This can also be seen through Figure 1.6 where we can see the asynchronous reset being called behaving as expected.

Exercise 5B:

Figure 2.1 – Counter_4bit.v Script

/************************************

*   FILE:                   Counter_4bit.v

*   AUTHOR:             Mohan Francis, Josh Ratificar

*   Class:              Gr.3 CpE 3101L Introduction to HDL

*   Group/Schedule      Friday, 7:30 AM to 10:30 AM

*   Description:        Counter_4bit.v file

************************************/

//Notes:

//nReset   |   Clk   |  Load  |  Count_en  |  Up  |   Counter Operation

//    0    |    X    |    X   |      X     |  X   |    Reset to 0

//    1    |        |    1   |      X     |  X   |    Load inputs (parallel load)

//    1    |        |    0   |      0     |  X   |    No change

//    1    |        |    0   |      1     |  0   |    Count down

//    1    |        |    0   |      1     |  1   |    Count up

module Counter_4bit(

input Clk, nReset, Load, Count_en, Up,

input [3:0] Count_in,

output reg [3:0] Count_out

);

always @(negedge Clk, negedge nReset)

begin

if(nReset == 0) // Checking for Asynchronous Reset

Count_out <= 4'b0000;

else if(Load == 1) // Status of Load Pin

Count_out <= Count_in;

else if(Count_en == 1) // Counter Enable

begin

if(Up == 1) // If up is high, then count up

Count_out <= Count_out + 1;

else // If up is low, then we should count down

Count_out <= Count_out - 1;

end

end

endmodule

Figure 2.2 – Counter_4bit.v Analysis and Elaboration Test Results

Figure 2.3 – Counter_4bit RTL View Output

Figure 2.4 – tb_ Counter_4bit.v Script for Testing Module

/************************************

*   FILE:               tb_Counter_4bit.v

*   AUTHOR:             Mohan Francis, Josh Ratificar

*   Class:              Gr.3 CpE 3101L Introduction to HDL

*   Group/Schedule      Friday, 7:30 AM to 10:30 AM

*   Description:        Testbench for Counter_4bit.v file

************************************/

`timescale 1 ns / 1 ps

module tb_Counter_4bit();

reg Clk, nReset, Load, Count_en, Up;

reg [3:0] Count_in;

wire [3:0] Count_out;

Counter_4bit UUT(

.Clk(Clk),

.nReset(nReset),

.Load(Load),

.Count_en(Count_en),

.Up(Up),

.Count_in(Count_in),

.Count_out(Count_out)

);

// Clock generation

always #5 Clk = ~Clk;

// Initial block for stimulus generation

initial begin

$display("Test Bench | Counter_4bit...");

$display("Initialize inputs...");

Clk = 0; nReset = 1; Load = 0; Count_en = 0; Up = 0; Count_in = 4'b0000; #10

$display("Loading Counter with 15d");

Load = 1; #10

Load = 0; #10

Count_in = 4'b1111; #10; // 15

$display("Enabling Counting");

Count_en = 1; #10

$display("Countdown:");

// Counting down

#10 Up = 0; Load = 0; Count_en = 1;  // 14

Count_en = 1; #10; // 13

Count_en = 1; #10; // 12

Count_en = 1; #10; // 11

Count_en = 1; #10; // 10

Count_en = 1; #10; // 09

Count_en = 1; #10; // 08

Count_en = 1; #10; // 07

Count_en = 1; #10; // 06

Count_en = 1; #10; // 05

Count_en = 1; #10; // 04

Count_en = 1; #10; // 03

Count_en = 1; #10; // 02

Count_en = 1; #10; // 01

Count_en = 1; #10; // 00

// Counting up

Up = 1; Count_en = 1; #10;  // 01

Count_en = 1; #10; // 02

Count_en = 1; #10; // 03

Count_en = 1; #10; // 04

Count_en = 1; #10; // 05

Count_en = 1; #10; // 06

Count_en = 1; #10; // 07

$display("Resetting");

nReset = 0; #10;

nReset = 1; #10;

$stop;

end

initial begin

$display("Test Bench | Counter_4bit...");

$monitor("Time = %2d ns | Clk = %b | nReset = %b | Load = %b | Count_en = %b | Up = %b | Count_in = %b | Count_out = %d", $time, Clk, nReset, Load, Count_en, Up, Count_in, Count_out);

end

endmodule

Figure 2.5 – tb_ Counter_4bit RTL Simulation Output

Figure 2.6 – tb_ Counter_4bit Test Bench Monitor Output (Annotations to Figure 2.5)

Discussion of Results (Exercise 5B)

The 4-bit counter operates as expected as observed in Figure 2.5. Through the test bench, we confirmed that the circuit can switch to countdown mode and as well count up mode. The asynchronous active low reset as well works as expected. There is no change when the clock is low which is also the behaviour that is desired. The Figure 2.6 “$monitor” command really makes it easier to see that this behaviour is consistent.